Liquid Crystal Devices for Optical Bio-Imaging Systems

- Abdulhalim

Department of Electro optic Engineering and the Ilse Katz Institute for Nanoscale Science and Technology, Ben Gurion University of the Negev, Beer Sheva 84105, Israel

Liquid crystal devices are under extensive study for photonic and optical non-display applications.  One of the important areas where they can significantly improve applications is in optical imaging in which they can function as spatial light modulators for wavefront correction, tunable filtering, tunable focusing and polarization control with their distinct advantage of being miniature.  Recently we have been investigating liquid crystal devices for this special purpose for biomedical imaging applications.  Several novel devices will be described and their integration into the specific system:

- Discrete wavelength tunable filter.

- Continuous high dynamic tunable filter.

- Continuous wavelength independent polarization rotator,.

Spectropolarimetric imaging system that uses these devices will be described with application for skin cancer detection.  In addition using a fast liquid crystal phase modulator we developed a high resolution full field optical coherence microscope capable of observing 3D microbiological structures as small as 0.4x0.4x1.0 μm3 (xyz) using quasi monochromatic light,.

References:

Avner Safrani, Ofir Aharon, Shahar Mor, Ofer Arnon, Lior Rosenberg and I. Abdulhalim, Skin biomedical optical imaging system using dual wavelength polarimetric control with liquid crystals, Journal of Biomedical Optics 15, 026024-8p (2010).

Ofir Aharon and I. Abdulhalim, Liquid crystal tunable filter with extended free spectral range, Optics Express, 17, 11426-33 (2009).

Avner Safrani and I. Abdulhalim, Liquid crystal polarization rotator and a tunable polarizer, Optics Letters, 34, 1801-3 (2009).

Ofir Aharon, I. Abdulhalim, Liquid crystal wavelength independent continuous polarization rotator, Optical Engineering 49, 034002-4p (2010).

Ofir Aharon, I. Abdulhalim, Ofer Arnon, Lior Rosenberg, Victor Dyomin, Eldad Silberstein, Differential optical spectropolarimetric imaging system assisted by liquid crystal devices for skin imaging, J. Biomedical Optics, 16(8), 086008-12p (2011).

Avner Safrani and I. Abdulhalim, Spatial coherence effect on layers thickness determination in narrowband full field optical coherence tomography, Applied Optics 50, 3021-27 (2011).

Avner Safrani and I. Abdulhalim, Ultra High Resolution Full Field Optical Coherence Tomography Using Spatial Coherence Gating and Quasi Monochromatic Illumination, Opt. Lett. 37, 458 (2012).